| ### CHECK LIST 4 - PRIOR CARGO TRANSFER Discharging Ship’s Name: |  |  |
| --- | --- | --- |
| Receiving Ship’s Name: |  |  |
| Name of the Designated PIC of STS operation: |  |  |
| Name of the STS Superintendent if Different from PIC: |  |  |
| Date and Location of Transfer: |  |  |
|  | Checked | Remarks |
| - Checklist 3 has been satisfactory completed |  |  |
| - A ship/shore safety checklist Form 2.3.3 has been satisfactorily completed and agreed between ships |  |  |
| - Required regional checklists (if applicable) have been completed |  |  |
| - Procedures for transfer of personnel have been agreed. - (Transfer of personnel using ship’s crane is not permitted) |  |  |
| - If used, the gangway/ladder is correctly positioned and well secured |  |  |
| - Inter-ship communication systems, including back-up, are agreed and tested |  |  |
| - Emergency signals and operation shutdown procedures are agreed |  |  |
| - The engine room will be manned as required throughout transfer and main engine is maintained on standby or on short notice of readiness |  |  |
| - A bridge watch and/or an anchor watch is established |  |  |
| - Officers in charge of the cargo transfer on both ships are identified and details have been exchanged and posted |  |  |
| - A deck watch has been established to pay particular attention to moorings, fenders, cargo operation and overside |  |  |
| - The cargo loading/unloading sequence has been agreed between ships (refer Form 2.3.2 – Bulk Cargo loading/unloading sequence) |  |  |
| - Arrangements have been made for the regular exchange of information on quantities of cargo transferred |  |  |
| - Arrangements have made for draft checks and for joint survey e.g. boat or ladder |  |  |
| - The topping-off/trimming rate and sequence has been agreed |  |  |
| - The procedure for stopping cargo transfer operation is agreed and recorded |  |  |
| - Ballasting and deballasting arrangements are agreed |  |  |
| - Stopping of the cargo operation in case of excessive movement of cargo crane hook due to rolling has been agreed and recorded |  |  |
| - For underway operation, the course and speed adjusted to reduce any movement of vessel due to sea and swell. |  |  |
| - Messengers are prepared and positioned ready for unmooring in accordance with the unmooring plan |  |  |
| - Security information has been exchanged and, if required, a Declaration of Security has been completed |  |  |
| - The other ship has been advised that checklist 4 is satisfactorily completed |  |  |
| FOR DISCHARGING SHIP/RECEIVING SHIP (Delete as appropriate) |  |  |
| Name: |  |  |
| Rank: |  |  |
| Signature: Date: |  |  |