| VESSEL |  |  | DATE: |  | PORT |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Laminated copy of the checklist shall be maintained in ships office and compliance will be recorded in deck log book |  |  |  |  |  |  |  |  |  |
| S.NO | Description |  |  |  |  |  | YES | NO | NA |
| Section A - During Operations |  |  |  |  |  |  |  |  |  |
| 1 | Crane movements for slew, luff and hoist are smooth, and controllable without undue sudden motions or abrupt stop. |  |  |  |  |  | ☐ | ☐ | ☐ |
| 2 | Cranes operating properly and there are no abnormal sound/vibrations |  |  |  |  |  | ☐ | ☐ | ☐ |
| 3 | Lifting operations stopped if weather conditions make it unsafe to continue |  |  |  |  |  | ☐ | ☐ | ☐ |
| 4 | Cranes not operated outside their design limits regarding SWL, list, trim or dynamic movement of the vessel |  |  |  |  |  | ☐ | ☐ | ☐ |
| 5 | Crane operators are not smoking anywhere inside the crane |  |  |  |  |  | ☐ | ☐ | ☐ |
| 6 | Unsafe practices or poor operations brought to the attention of the super cargo / stevedore foreman / vessel operators |  |  |  |  |  | ☐ | ☐ | ☐ |
| 7 | Wires not making contact with the coamings |  |  |  |  |  | ☐ | ☐ | ☐ |
| 8 | Personnel not standing under suspended loads |  |  |  |  |  | ☐ | ☐ | ☐ |
| 9 | Crane drivers not putting excessive shock loads on the crane wires |  |  |  |  |  | ☐ | ☐ | ☐ |
| 10 | Grab cable and reeling drum (were fitted) working smoothly |  |  |  |  |  | ☐ | ☐ | ☐ |
| 11 | Ship staff not operating cranes ( Cranes to be operated only by stevedores) |  |  |  |  |  | ☐ | ☐ | ☐ |
| Section B - During breaks ( To the extent practical ) |  |  |  |  |  |  |  |  |  |
| 12 | Crane hydraulic systems checked for leaks |  |  |  |  |  | ☐ | ☐ | ☐ |
| 13 | Wires inspected for damages ( check for fish hooks / stands of broken wire in the drum area or on deck) |  |  |  |  |  | ☐ | ☐ | ☐ |
| 14 | Crane hydraulics switched off whenever there is no crane driver in the cab |  |  |  |  |  | ☐ | ☐ | ☐ |
| 15 | Greasing of grab is carried out as per maker manual |  |  |  |  |  | ☐ | ☐ | ☐ |
| 16 | Confirm wire is on the sheave and protected by wire guards ( Wire not jumped off from sheave) |  |  |  |  |  | ☐ | ☐ | ☐ |
| 17 | Cargo sheaves checked for grooving and wear |  |  |  |  |  | ☐ | ☐ | ☐ |
| Section C- After operations are completed |  |  |  |  |  |  |  |  |  |
| 18 | Jib is resting properly on the parking stand |  |  |  |  |  | ☐ | ☐ | ☐ |
| 19 | Lashings of Jib and crane hook are in place and tight |  |  |  |  |  | ☐ | ☐ | ☐ |
| 20 | All levers are in "Normal" position |  |  |  |  |  | ☐ | ☐ | ☐ |
| 21 | Power is switched off and engine room notified |  |  |  |  |  | ☐ | ☐ | ☐ |
| 22 | Space heaters (Electric motor heater) Kept ON |  |  |  |  |  | ☐ | ☐ | ☐ |
| 23 | Cabin heaters put OFF |  |  |  |  |  | ☐ | ☐ | ☐ |
| 24 | Cabin windows closed and door secured |  |  |  |  |  | ☐ | ☐ | ☐ |
| 25 | Check for any damages / missing items and issue protest as soon as possible |  |  |  |  |  | ☐ | ☐ | ☐ |