| VESSEL |  | DATE: |  | CNO: |  |
| --- | --- | --- | --- | --- | --- |

| S.NN | DESCRIPTION | YES | NO | NA |
| --- | --- | --- | --- | --- |
|  | CARGO HATCH |  |  |  |
| 1 | Coaming drains not clogged and clear and all non-return valves working | ☐ | ☐ | ☐ |
| 2 | No leaks on hydraulic cylinders. Covers for hydraulic cylinder in good condition and effective. Spare covers on board | ☐ | ☐ | ☐ |
| 3 | Hatch covers externally examined and panel tops free of damage, wastage and holes and securing arrangements in good order | ☐ | ☐ | ☐ |
| 4 | Cleats all in place, not damaged, rods straight, washer good, surrounding steel work in good condition. All cleats correctly applied | ☐ | ☐ | ☐ |
| 5 | Bearing pads, chocks, or support blocks free of wear or damage. Bearing pads are in contact and providing correct support and rubber seal compression | ☐ | ☐ | ☐ |
| 6 | Adjacent hatch cover panels are in proper alignment | ☐ | ☐ | ☐ |
| 7 | Hatch covers aligned and is the height of the hatch cover side uniform showing the cover rubber effectively sitting on the compression bar | ☐ | ☐ | ☐ |
| 8 | Side clips making contact to lock the covers together | ☐ | ☐ | ☐ |
| 9 | Coaming tops free from grooving or wear and rust scale | ☐ | ☐ | ☐ |
| 10 | Hold ventilators free of damage and wastage and closing correctly | ☐ | ☐ | ☐ |
| 11 | Shim Plates to be added between wedge and base part of stopper in case of wearing of wedge and insufficient rubber compression in meeting joint | ☐ | ☐ | ☐ |
| 12 | CO2 system valves to hatches are operational and marked. | ☐ | ☐ | ☐ |
| 13 | No signs of water ingress in any hold | ☐ | ☐ | ☐ |
|  | COFFERDAMS |  |  |  |
| 14 | All cofferdams soundings checked to ensure no leaking steam or fuel lines. | ☐ | ☐ | ☐ |
|  | LOADLINE |  |  |  |
| 15 | All weather tight doors on deck in good condition; (check for corrosion, buckling of door and hinges, deterioration of gasket retaining channel, missing/ deteriorated gasket and missing /frozen/corroded dogs/cleats). For checking the gasket condition, the light should not be visible inside when door is closed | ☐ | ☐ | ☐ |
| 16 | Booby hatch rubber packing and securing arrangements in good condition and sealing effectively. Closing mechanism (where fitted) is not jammed and free to operate. The light should not be visible inside when door is closed. (Do not open fumigated spaces) | ☐ | ☐ | ☐ |
| 17 | Rubber packing on weather tight doors are not painted | ☐ | ☐ | ☐ |
| 18 | Metallic caps for sounding pipes are not missing and intact and thread allow caps to be secured in place | ☐ | ☐ | ☐ |
| 19 | Butterfly nuts/ Locking pins/ securing chains as applicable for ventilator flaps, sky light, booby hatch covers, rope locker etc are not wasted, intact and not missing. Ensure locking pins are secured and fit in correctly | ☐ | ☐ | ☐ |
| 20 | Sounding pipes/vent pipes/ air pipes are clearly marked indicating the space they serve | ☐ | ☐ | ☐ |
| 21 | Forepeak store bilges kept clear of debris for emergency pumping out of bilges | ☐ | ☐ | ☐ |
| 22 | All ballast tank covers are fully bolted | ☐ | ☐ | ☐ |
| 23 | Freeing ports / draining arrangements / scuppers in good order | ☐ | ☐ | ☐ |
| 24 | All air pipes/vents externally examined for damage and wastage, clearly marked, and show date of last overhaul (month/year). | ☐ | ☐ | ☐ |
| 25 | Railings, cat walks and safety lines are well maintained, not corroded, holed, or wasted | ☐ | ☐ | ☐ |
| 26 | Timber fittings where fitted are in good condition and not damaged | ☐ | ☐ | ☐ |
| 27 | Plimsol lines are clearly visible and marked with the contrast colour paint. (To check when vessel at anchorage or at berth) | ☐ | ☐ | ☐ |
|  | MISCELLANEOUS |  |  |  |
| 28 | No spillage of grease from crane wires on the deck structures | ☐ | ☐ | ☐ |
| 29 | Any portable ladder on deck kept only on gratings/dunnage, properly secured, and covered | ☐ | ☐ | ☐ |
| 30 | Gangway motors properly covered | ☐ | ☐ | ☐ |
| 31 | All save all trays have plugs which are secured by chains. Threads in save all tray not wasted, and plugs tried out for proper closing | ☐ | ☐ | ☐ |
| 32 | Garbage stowing and securing in good order as per garbage management plan. Garbage drums kept on wooden pallets and not directly on the deck. Check drums for corrosion in its base. Drums should have lids properly sealing the drums without any gaps | ☐ | ☐ | ☐ |
| 33 | Crane securing and blocks are in good condition and swivels are free with reference marking visible. | ☐ | ☐ | ☐ |
| 34 | Helicopter pad access ladder in good condition | ☐ | ☐ | ☐ |
| 35 | Windlass /winches brakes in good condition and there are no leaks or operational issues | ☐ | ☐ | ☐ |
| 36 | Pilot ladder securing equipment, bulwark ladder, and pilot ladder securing arrangements in good condition | ☐ | ☐ | ☐ |

Additional Comments: