| Vessel Name |  | Date |  |
| --- | --- | --- | --- |
| Work location |  | Description of hot work |  |
| Equipment used for hot work |  | Personnel carrying hot work |  |
| Officer responsible for hot work |  | Officer responsible for safety |  |
| Name of cargo on board |  | Sea/ Port / Anchor |  |

| S/N | ITEM | YES | NO | N/A |
| --- | --- | --- | --- | --- |
| 1 | Risk assessment /Tool box meeting completed . All personnel involved briefed on the plan , safety precautions,nature of work including simultaneous operations prior commencement. | ☐ | ☐ | ☐ |
| 2 | If hot work is in enclosed space ,has enclosed space entry permit been issued | ☐ | ☐ | ☐ |
| 3 | Company approval obtained and comments complied with | ☐ | ☐ | ☐ |
| 4 | Hot work area /adjacent areas free from combustible materials, flammable liquids , oil etc | ☐ | ☐ | ☐ |
| 5 | Space and adjacent / surrounding areas clean and rendered safe and gas free ( 0 % LEL) | ☐ | ☐ | ☐ |
| 6 | Hot Work area more than 500 mm from fuel oil tank deck or bulkhead. | ☐ | ☐ | ☐ |
| 7 | Hotwork will not cause heat transfer throughout the bulkhead of bunker tank | ☐ | ☐ | ☐ |
| 8 | Additional fire protection like fire extinguishers/hoses provided onsite and fully operational | ☐ | ☐ | ☐ |
| 9 | Fire Main pressurised | ☐ | ☐ | ☐ |
| 10 | Fire detection system isolated | ☐ | ☐ | ☐ |
| 11 | Fire watch crew designated for full duration in the area of hot work and adjacent areas | ☐ | ☐ | ☐ |
| 12 | Space segregated and equipments or pipelines gas free / blanked / free of liquid/sludge | ☐ | ☐ | ☐ |
| 13 | Applicable valves shut / lashed |  |  |  |
| 14 | Arrangements made for the space to be continuously ventilated throughout the period of occupation and during work breaks | ☐ | ☐ | ☐ |
| 15 | Space tested and found safe for entry Note: In order to obtain a representative cross-section of the space’s atmosphere, samples should be taken from several levels and through as many openings as possible. Ventilation should be stopped for about 10 minutes before the pre-entry atmosphere tests are taken | ☐ | ☐ | ☐ |
| 16 | Initial checks prior commencement of work using gas meter: Oxygen (actual reading) % Vol (20.9%) Hydrocarbon %LFL (Company requirement 0 %) Checked By (Name) ___________ Checked Time_______ Date : ______________ | ☐ | ☐ | ☐ |
| 17 | Arrangements been made for frequent atmosphere checks while the space is occupied and after work breaks. At intervals of minutes (Not exceeding 30 minutes) | ☐ | ☐ | ☐ |
| 18 | Space protected from accidental entry. Barriers/warning signs in place | ☐ | ☐ | ☐ |
| 19 | Safe and adequate access / egress established | ☐ | ☐ | ☐ |
| 20 | Illumination adequate (Torches / portable lights etc) | ☐ | ☐ | ☐ |
| 21 | Officer of the Watch (bridge, engine room) been advised | ☐ | ☐ | ☐ |
| 22 | Lines of communications been clearly established, tested and understood by all concerned and emergency signals agreed | ☐ | ☐ | ☐ |
| 23 | Welding and other equipments tested and in good working order and properly earthed | ☐ | ☐ | ☐ |
| 24 | All personnel wearing proper PPE and suitably equipped | ☐ | ☐ | ☐ |
| 25 | Emergency scenarios taken into consideration and response plan discussed Emergency scenarios e.g. Fire, burn injury, electrocution, working in enclosed space etc | ☐ | ☐ | ☐ |
| 26 | Written approvals from terminal / port authorities obtained if vessel is in port | ☐ | ☐ | ☐ |
| 27 | Equipments isolated electrically | ☐ | ☐ | ☐ |
| Lock Out Tag Out (LOTO) for isolation |  |  |  |  |
| 1. |  |  |  |  |
| 2. |  |  |  |  |
| Additional precautions |  |  |  |  |
| 1. |  |  |  |  |
| 2. |  |  |  |  |
| Emergency response plan, enter Emergency Response Plans discussed under item 25. |  |  |  |  |
| 1. |  |  |  |  |
| 2. |  |  |  |  |
| 3. |  |  |  |  |

Atmosphere readings to be monitored for work area , adjacent tanks and spaces prior commencement of hot work and at intervals of not more than 30 Minutes during hot work

| Date | Time | % O2 Vol | HC % LEL | Remarks (After lunch/tea breaks/repetitive intervals Not exceeding 30 minutes) |
| --- | --- | --- | --- | --- |

| I understand the work to be carried out, safety measures necessary. The above mentioned points have been checked and the conditions at the site, or of the equipment to be worked on, are safe for the work to be undertaken and I am satisfied that it is safe to commence work |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Officer responsible for hot work | Name |  |  |  | Rank |  | Signature |  |  |  |  |  |
| Officer responsible for safety | Name |  |  |  | Rank |  | Signature |  |  |  |  |  |
| Crew members / team |  |  |  |  |  |  |  |  |  |  |  |  |
|  | I understand the work to be carried out. I have been briefed on safety measures and I am satisfied to commence work. |  |  |  |  |  |  |  |  |  |  |  |
|  | Name |  |  |  | Rank |  | Signature |  |  |  |  |  |
|  | Name |  |  |  | Rank |  | Signature |  |  |  |  |  |
| Officer authorising work |  |  |  |  |  |  |  |  |  |  |  |  |
| Master / Chief Engineer |  |  | Name: Signature: |  |  |  |  |  |  |  |  |  |
| Date: |  |  | Time authorised: |  |  |  |  |  |  |  |  |  |
| Period of Validity (Period of validity to commence only after permit has been authorised) . Max Validity of 8 hours or Permit remains valid only as long as the permit conditions are met |  |  |  |  |  |  |  |  |  |  |  |  |
| VALIDITY FROM |  |  |  | DATE: |  |  | TIME: |  |  |  |  |  |
| VALIDITY TO: |  |  |  | DATE: |  |  | TIME: |  |  |  |  |  |
| Completion of job (To be completed by the responsible person) |  |  |  |  |  |  |  |  |  |  |  |  |
| The work has been completed and all persons under my supervision, materials ,equipment and notices have been withdrawn. All items re-secured and lock /tag out removed. Fire detection system restored. The work area and equipment has been left in a safe condition. All persons involved advised and permit has ended. ☐ |  |  |  |  |  |  |  |  |  |  |  |  |
| Reason for closing the permit |  |  |  |  |  |  |  |  |  |  |  |  |
| Work Completed |  | ☐ | Permit expired (in excess of 8 hrs) |  | ☐ | Change of Circumstance Specify in detail: |  | ☐ | Work Suspended | ☐ |  |  |
| Permit Closure |  |  |  |  |  |  |  |  |  |  |  |  |
| Permit Closed by |  | Name |  |  | Signature |  |  | Date |  | Time |  |  |
|  |  |  |  |  |  |  |  |  | dd/mm/yyyy |  |  | hh/mm |

Note: This permit is rendered invalid if any of the conditions noted in the checklist change