# WATCH ARRANGEMENTS - COMPOSITION – FITNESS

The Master is responsible for the safety of navigation, and for setting the watches so as to be adequate and fit at all times to meet the prevailing circumstances in compliance with SOLAS, STCW, LOCAL REGULATIONS and COLREGS.

The minimum bridge manning levels to be complied with are tabulated below. Master can enhance manning levels as per his discretion having given due consideration to section 2.3 of Nautical Manual - 4.0. Bridge Organisation

| MINIMUM BRIDGE MANNING LEVELS TO BE COMPLIED WITH |  |  | PILOT | CAPT / CNO | OOW | HELMSMAN | LOOK OUT |
| --- | --- | --- | --- | --- | --- | --- | --- |
| SCENARIO | CONDITIONS | MANNINGLEVEL |  |  |  |  |  |
| PASSAGE COASTAL | DAY | 1 |  |  | x |  | Optional as per Master’s discretion |
| PASSAGE COASTAL | NIGHT | 2 |  |  | x |  | x |
| PASSAGE OCEAN | DAY | 1 |  |  | x |  |  |
| PASSAGE OCEAN | NIGHT | 2 |  |  | x |  | x |
| ANCHOR | DAY | 1 |  |  | x |  | Optional as per Master’s discretion |
| ANCHOR | NIGHT | 2 |  |  | x |  | x |
| RESTRICTED / CONFINED WATERS | DAY or NIGHT | 2 |  |  | x | Optional as per Master’s discretion | x |
| MANOEUVRING (Entering / Leaving Port / Anchoring) | DAY or NIGHT | 4 |  | x | x | x | x |
| HEAVY TRAFFIC / CONGESTED WATERS | DAY or NIGHT | 4 |  | x | x | x | x |
| HEAVY WEATHER | DAY | 3 |  | x | x | x |  |
| HEAVY WEATHER | NIGHT | 4 |  | x | x | x | x |
| RESTRICTED VISIBILITY | DAY or NIGHT | 4 |  | x | x | x | x |
| PIRACY WATCH / HRA TRANSIT | DAY or NIGHT | 3 |  |  | x | x | x |
| PILOTAGE (Entering / Leaving Port / Channel Transit) | DAY or NIGHT | 5 | x | x | x | x | x |