| VESSEL |  |  | DATE: | Click or tap to enter a date. | VOY NO: |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| A laminated copy of the checklist shall be maintained in wheelhouse and compliance will be recorded in deck logbook. |  |  |  |  |  |  |  |  |
| 1.0 | Passage Plan |  |  |  |  |  | YES | NA |
| 1.1 | Pre-arrival documentation complete and sent |  |  |  |  |  |  |  |
| 1.2 | Passage plan updated with additional information received since departure |  |  |  |  |  |  |  |
| 1.3 | Any restrictions on draft, trim, speed entry times etc |  |  |  |  |  |  |  |
| 1.4 | Passage plan prepared till berth. UKC sheet attached with passage plan. Company informed if vessel does not comply with the UKC policy. |  |  |  |  |  |  |  |
| 1.5 | Updated passage plan checked and approved by the Master |  |  |  |  |  |  |  |
| 1.6 | Updated passage plan briefed to the Bridge Team |  |  |  |  |  |  |  |
| 1.7 | All charts /publications corrected to latest Notices to Mariners (WEEK N0: ) |  |  |  |  |  |  |  |
| 1.8 | “NO GO” point been established & contingency anchoring position identified |  |  |  |  |  |  |  |
| 1.9 | Route plotted on up to date and appropriate scale charts (paper or electronic) |  |  |  |  |  |  |  |
| 1.10 | Updated route displayed on ECDIS and other electronic navigation aids |  |  |  |  |  |  |  |
| 1.11 | Cargo/ballast adjustment required to bring vessel to even keel / maximize UKC |  |  |  |  |  |  |  |
| 1.12 | Air draft/overhead clearance calculated and sufficient for bridges/overhead cable |  |  |  |  |  |  |  |
| 1.13 | Applicable local area weather and navigation warnings applied on charts |  |  |  |  |  |  |  |
| 2.0 | Equipment Checks (Tested and Ready for Use) |  |  |  |  |  | YES | NA |
| 2.1 | All clocks synchronised (Bridge, Engine room etc) |  |  |  |  |  |  |  |
| 2.2 | Power available for windlass/winches/cranes/gangway etc |  |  |  |  |  |  |  |
| 2.3 | ECDIS-Correct contours setting done/sensor feed from various equipment verified |  |  |  |  |  |  |  |
| 2.4 | Echo sounder (time/date/position marked), left in operation with sufficient paper |  |  |  |  |  |  |  |
| 2.5 | Course recorder (time/date/position marked), left in operation with sufficient paper |  |  |  |  |  |  |  |
| 2.6 | GPS (correct datum WGS 84 in use) and waypoint entered till berth |  |  |  |  |  |  |  |
| 2.7 | Emergency engine stops |  |  |  |  |  |  |  |
| 2.8 | Bridge and engine room telegraphs |  |  |  |  |  |  |  |
| 2.9 | Engine movement recorder kept “ON “with sufficient paper and annotated |  |  |  |  |  |  |  |
| 2.10 | Engine(s)/propulsion tried (ahead and astern) and prepared for manoeuvring |  |  |  |  |  |  |  |
| 2.11 | All internal vessel control communications (bridge to E/Room, mooring stations, steering gear etc) and vessel control alarms |  |  |  |  |  |  |  |
| 2.12 | Radio communications equipment including GMDSS, and VHF tested |  |  |  |  |  |  |  |
| 2.13 | Walkie-talkies tested and spare batteries charged and in readiness |  |  |  |  |  |  |  |
| 2.14 | Gyro/magnetic compass and repeaters including steering gear repeater synchronised and error checked. Magnetic compass periscope clean. Tilt the spare magnetic compass bowl and check if any bubble is present. The magnetic compass on the monkey island shall also be tilted and checked for bubble. |  |  |  |  |  |  |  |
| 2.15 | Both radars including ARPA/plotting functions ON and heading aligned |  |  |  |  |  |  |  |
| 2.16 | Navigation lights, shapes and sound signals including, whistle, fog bell & gong. Clean the glass of lights if found dusty and dirty. The forward and aft whistle tested on Manual as well as Electric mode. Talk back system tested from all locations including bridge wings where applicable |  |  |  |  |  |  |  |
| 2.17 | NUC, signal lights, deck lights, over side lights, search lights & emergency lights |  |  |  |  |  |  |  |
| 2.18 | Navigational light failure alarm tested |  |  |  |  |  |  |  |
| 2.19 | Ancillary bridge equipment (e.g. binoculars, Azimuth mirror, thermometer) |  |  |  |  |  |  |  |
| 2.20 | RPM and ROT indicators |  |  |  |  |  |  |  |
| 2.21 | Signalling equipment, flags (G, H, Q, B, courtesy etc) and signal lamps |  |  |  |  |  |  |  |
| 2.22 | Steering gear (Checklist NAV B1) |  |  |  |  |  |  |  |
| 2.23 | Thrusters (Covers removed from bow thruster control boxes and clean within) |  |  |  |  |  |  |  |
| 2.24 | Navtex / SAT C- EGC receiver selected to the correct Navarea |  |  |  |  |  |  |  |
| 2.25 | Weather fax programmed to required station where applicable |  |  |  |  |  |  |  |
| 2.26 | Visual checks of EPRIB and SART |  |  |  |  |  |  |  |
| 2.27 | AIS fixed or static information checked if correct / voyage related information updated / dynamic information from sensors checked for accuracy / navigation status updated /Power changed to low power if required. |  |  |  |  |  |  |  |
| 2.28 | VDR /SVDR operational and no-fault alarms |  |  |  |  |  |  |  |
| 2.29 | Bridge Navigational Watch Alarm System (BNWAS) operational |  |  |  |  |  |  |  |
| 2.30 | Window wipers/clear view screens checked. Windows and bridge wings cleaned |  |  |  |  |  |  |  |
| 2.31 | Speed and distance log checked |  |  |  |  |  |  |  |
| 2.32 | LRIT |  |  |  |  |  |  |  |
| 2.33 | Illumination of equipment (RPM Indicator, Repeaters, Steering Lighting etc) |  |  |  |  |  |  |  |
| 2.34 | Standby or emergency generator, for as long as necessary to show proper functioning, including steady state temperature and pressure readings |  |  |  |  |  |  |  |
| 2.35 | Storage batteries for emergency lighting and power systems in vessel control and propulsion machinery spaces |  |  |  |  |  |  |  |
| 3.0 | Before Arrival |  |  |  |  |  | YES | NA |
| 3.1 | Anchors cleared and ready for use and anchoring party called |  |  |  |  |  |  |  |
| 3.2 | Bridge Team ready / Helmsman on bridge well in time |  |  |  |  |  |  |  |
| 3.3 | Cargo details available |  |  |  |  |  |  |  |
| 3.4 | Engine room ready |  |  |  |  |  |  |  |
| 3.5 | If available, use more than one steering gear power unit |  |  |  |  |  |  |  |
| 3.6 | Manual steering engaged |  |  |  |  |  |  |  |
| 3.7 | Mooring stations manned and ready and heaving line in readiness. No added weights on monkey fist |  |  |  |  |  |  |  |
| 3.8 | Pressure on fire main |  |  |  |  |  |  |  |
| 3.9 | Stability and draught information verified and available |  |  |  |  |  |  |  |
| 3.10 | Watertight doors closed |  |  |  |  |  |  |  |
| 4.0 | Port and Pilotage Requirements |  |  |  |  |  | YES | NA |
| 4.1 | Master/Pilot information exchange checklist completed (Form 1.5.2A) |  |  |  |  |  |  |  |
| 4.2 | Pilot Card prepared (Form 1.5.2A) |  |  |  |  |  |  |  |
| 4.3 | Pilot boarding time confirmed |  |  |  |  |  |  |  |
| 4.4 | Pilot boarding arrangements rigged properly as per the poster “Required boarding arrangement for pilot. Lifebuoy equipped with self-igniting light and heaving line kept at hand and ready for immediate use |  |  |  |  |  |  |  |
| 4.5 | Port, pilot and VTS channels monitored |  |  |  |  |  |  |  |
| 4.6 | Port, VTS and Pilot advised of any special requirements that ship may have |  |  |  |  |  |  |  |
| 4.7 | Preparations for pilotage complete (NAV B8) |  |  |  |  |  |  |  |
| 5.0 | Other |  |  |  |  |  | YES | NA |
| 5.1 | Times and height of tides noted, direction and velocity of current checked |  |  |  |  |  |  |  |
| 5.2 | ETA been sent with all relevant information (e.g., details of dangerous/hazardous goods carried, ballast exchange information etc) to agents, port authority etc |  |  |  |  |  |  |  |
| 5.3 | Reporting arrival information (ETA, VTS, AMVER etc) complied with |  |  |  |  |  |  |  |
| 5.4 | Local port/environmental requirements requested from agents and complied |  |  |  |  |  |  |  |
| 5.5 | Master / OOW aware of Standby time and position of SBE marked on chart |  |  |  |  |  |  |  |
| 5.6 | Engine room been informed at least one hour prior to Standby Manoeuvring |  |  |  |  |  |  |  |
| 5.7 | Fuel changeover requirements (e.g., ECA) if applicable |  |  |  |  |  |  |  |
| 5.8 | All Alarm panels in order (Water ingress, bilge alarms etc as applicable) |  |  |  |  |  |  |  |
| 5.9 | Bridge Team fit for duty / Bridge manning level as per company requirement |  |  |  |  |  |  |  |
| 5.10 | Torches positioned as required and operational |  |  |  |  |  |  |  |
| 5.11 | Time and position of engaging hand steering recorded in Deck Logbook |  |  |  |  |  |  |  |
| 5.12 | Bridge movement book / bell book ready |  |  |  |  |  |  |  |
| 5.13 | Port documents and mail ready |  |  |  |  |  |  |  |
| 5.14 | Emergency Port Contact Sheet contained in the vessel’s SOPEP updated |  |  |  |  |  |  |  |
| 5.15 | Issues related to Port Security been addressed (stowaway precautions, security searches, positing of contact details as per SSP). Port security level confirmed |  |  |  |  |  |  |  |
| 5.16 | Measures taken to monitor suspicious crafts approaching in the vicinity |  |  |  |  |  |  |  |
| 5.17 | For US ports, tests required by 33 CFR 164.25 carried no more than 12 hrs before entering US waters and recorded in ships log |  |  |  |  |  |  |  |
| 5.18 | Lifebuoy with a self-igniting light and buoyant line kept for immediate use in vicinity of means of embarkation/disembarkation arrangement (accommodation ladder/ gangway) as per requirements of MSC Circular 1331 |  |  |  |  |  |  |  |
| 5.19 | Oil record Book /Garbage record book / Official logbook/ Ballast water record book / Fuel sulphur record book / GMDSS log / Deck log book entries up to date and signed |  |  |  |  |  |  |  |
| 5.20 | Inspection for Certificate free of FSMC (Flighted spongy moth complex / Asian gypsy moth) arranged before departure from Regulated Areas during Risk period (Refer Memo / Port Regulations for Bulletin on Joint Flighted Spongy Moth) |  |  |  |  |  |  |  |
| 6.0 | After Vessel Alongside |  |  |  |  |  | YES | NA |
| 6.1 | Bridge equipment put off/kept standby /changed to low power (As applicable) |  |  |  |  |  |  |  |
| 6.2 | BNWAS switched off and logged in deck log |  |  |  |  |  |  |  |
| 6.3 | Bridge squared up and doors locked |  |  |  |  |  |  |  |
| 6.4 | All emergency lights outside accommodation tested and damaged bulbs replaced immediately |  |  |  |  |  |  |  |