| VESSEL |  |  | DATE: | Click or tap to enter a date. | VOY NO: |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Laminated copy of the checklist shall be maintained in wheel house and compliance will be recorded in deck log book for each watch. |  |  |  |  |  |  |  |  |
| S.NO | Description |  |  |  |  |  | YES | NA |
|  | Appropriate scale charts available with route plotted |  |  |  |  |  |  |  |
|  | Courses been laid off well clear of obstructions |  |  |  |  |  |  |  |
|  | Advice/Recommendations in sailing directions considered |  |  |  |  |  |  |  |
|  | Ships draught in relation to available water depths considered |  |  |  |  |  |  |  |
|  | Proximity to shallow water and the effect of squat on UKC monitored |  |  |  |  |  |  |  |
|  | Tides and Currents (Set and Drift) calculated |  |  |  |  |  |  |  |
|  | Effects of weather and currents for the area understood and monitored particularly in areas prone to poor visibility |  |  |  |  |  |  |  |
|  | Bridge manning appropriate to maintain a proper look-out |  |  |  |  |  |  |  |
|  | Helmsman on bridge or available at immediate notice |  |  |  |  |  |  |  |
|  | Lookout on bridge or available at immediate notice |  |  |  |  |  |  |  |
|  | Lookout kept visually and by short range scanning on radar |  |  |  |  |  |  |  |
|  | ECDIS terminals setup correctly with route displayed |  |  |  |  |  |  |  |
|  | ECDIS : Adjust and use appropriate look ahead vector, check principal alarm settings, check appropriate layer in use. Check alarm buzzer is not mute |  |  |  |  |  |  |  |
|  | Echo sounder in operation and checked regularly. Echo sounder depth compared with the charted depth (in depths less than 100m) and comparison recorded in the deck logbook. |  |  |  |  |  |  |  |
|  | Available navigational aids and their accuracy considered |  |  |  |  |  |  |  |
|  | E/room informed that manoeuvring may be required and kept ready for immediate use |  |  |  |  |  |  |  |
|  | Anchor lashings removed and anchors ready for use [Master discretion] |  |  |  |  |  |  |  |
|  | Gyro/magnetic compass errors checked |  |  |  |  |  |  |  |
|  | Manual steering checked and ready for use (NAV B1). More than one steering gear power unit used, as appropriate |  |  |  |  |  |  |  |
|  | Measures taken to comply with environmental requirements and applicable pollution regulations including Particularly Sensitive Sea Areas (PSSA) requirement if applicable. Arrival port requirement pertaining to compliant fuel use, treated Sewage and Greywater discharge etc obtained by Master. All HOD been informed (E/R, Galley and Deck) |  |  |  |  |  |  |  |
|  | Emission control area (ECA) boundary including China and South Korea ECA plotted on chart and duty engineer informed at least three hours prior to entering the ECA. |  |  |  |  |  |  |  |
|  | Regulated California Waters (RCW) boundary 24 miles from the coast including islands plotted on chart and duty engineer informed at least three hours prior entering RCW for change over to distillate fuel (MDO and/or MGO) 0.1% Sulphur limit. |  |  |  |  |  |  |  |
|  | Panama Canal waters boundary plotted on chart and duty engineer informed at least three hours prior entering PC waters for change over to distillate fuel. |  |  |  |  |  |  |  |
|  | Duty engineer informed before entering Baltic Sea not to use the incinerator. |  |  |  |  |  |  |  |
|  | Duty engineer informed of EU Directive/Turkish port requirement (confirm from Master) regarding change over to 0.1% Sulphur limit fuel during stay at anchorage/berth. |  |  |  |  |  |  |  |
|  | MSI - Local/Coastal Warning Broadcasts being monitored (Navtex, Sat C etc.) and plotted as appropriate. ECDIS Route check carried out after plotting. |  |  |  |  |  |  |  |
|  | GPS positions accuracy verified by visual/radar bearing and distance lines of position (LOP) using terrestrial objects. Position fixing intervals / methods – As per passage plan. |  |  |  |  |  |  |  |
|  | All navigational aids being made use for position fixing and anti collision |  |  |  |  |  |  |  |
|  | Both Radar/ARPA kept ON.Parallel indexing used. Radar performance checked, recorded in deck logbook and radar heading line marker alignment checked |  |  |  |  |  |  |  |
|  | AIS accuracy of the target information checked by comparing radar ranges and bearing of the same target. |  |  |  |  |  |  |  |
|  | Ship security procedures understood and followed. Measures been taken to monitor boats, crafts approaching or operating in the vicinity (considering threat of piracy and armed robbery) |  |  |  |  |  |  |  |
|  | Vessel area reporting requirements understood and followed |  |  |  |  |  |  |  |
|  | Vessel routeing requirements / TSS requirements understood and followed |  |  |  |  |  |  |  |
|  | Light , shapes and sound signalling apparatus tested and kept ready |  |  |  |  |  |  |  |
|  | VHF on channel 16 & local channel. Nav warnings on VHF being noted |  |  |  |  |  |  |  |
|  | Daylight/Night time passing of danger points considered |  |  |  |  |  |  |  |
|  | Flow, volume and type of traffic likely to be encountered assessed, with particular regard given to fishing vessel concentrations as per passage plan and sailing directions and plans are in place to avoid passing within close proximity |  |  |  |  |  |  |  |
|  | Speed log accuracy checked by comparing with GPS |  |  |  |  |  |  |  |
|  | Additionally for Navigation in Narrow/Confined Waters |  |  |  |  |  | YES | NA |
|  | Comply with ROR Rule No. 9 when navigating in a narrow channel. |  |  |  |  |  |  |  |
|  | Pilot employed whenever the safe navigation of the vessel or local regulations make this necessary or mandatory. |  |  |  |  |  |  |  |
|  | Largest scale chart / ENC on board, suitable for the area and corrected with the latest available information used. |  |  |  |  |  |  |  |
|  | Master present on the bridge together with the OOW and a Helmsman as required by Bridge Manning Level. |  |  |  |  |  |  |  |
|  | The engine room manned and the engines on standby and immediately available for manoeuvring. Generators in auto load sharing mode. An additional generator placed on load. |  |  |  |  |  |  |  |
|  | Main engine tested well in time prior entering these waters. |  |  |  |  |  |  |  |
|  | Steering in manual mode. An additional steering gear power unit tested and kept readily available on immediate standby. |  |  |  |  |  |  |  |
|  | Anchors cleared and ready for immediate use. |  |  |  |  |  |  |  |
|  | A hand available forward station for emergency anchoring as required. |  |  |  |  |  |  |  |
|  | Speed regulated so as to comply with local speed restrictions and to avoid damage to shore installations, moored vessels and tows. |  |  |  |  |  |  |  |
|  | Safe under keel clearance maintained, making due allowance for squat, and care taken to ensure that predicted depths do in fact exist. Vessel movements planned to make the best use of tidal conditions. |  |  |  |  |  |  |  |
|  | Bank effect and suction phenomena considered when transiting in close proximity to a bank. |  |  |  |  |  |  |  |
|  | Wheel over marked adequately at turning points specially where large alteration of course is required. |  |  |  |  |  |  |  |
|  | Reducing speed or stopping vessel considered if in doubt of the ship’s position. |  |  |  |  |  |  |  |
|  | Available emergency anchorages marked on ECDIS. |  |  |  |  |  |  |  |
|  | Helmsman supervised closely during transit. |  |  |  |  |  |  |  |
|  | Compass error verified using transit bearing or leading lights as applicable. |  |  |  |  |  |  |  |
|  | Communication with VTS / Port Authority established (as required) for prevailing / expected traffic condition specially at turning points and in tight narrow passages or for any other pertinent information regarding navigation. |  |  |  |  |  |  |  |
|  | Other Checks |  |  |  |  |  | YES | NA |
| 56. |  |  |  |  |  |  |  |  |
| 57. |  |  |  |  |  |  |  |  |
| 58. |  |  |  |  |  |  |  |  |
| 59. |  |  |  |  |  |  |  |  |