| VESSEL |  |  | DATE: |  | VOY NO: |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Laminated copy of the checklist shall be maintained in wheel house and compliance will be recorded in deck log book for each watch. |  |  |  |  |  |  |  |  |
| S.NO | Description |  |  |  |  |  | YES | NA |
|  | Appropriate scale charts available with route plotted. All Charts and Nautical Publications to be used, been corrected up to date |  |  |  |  |  |  |  |
|  | All measures have been taken to comply with environmental requirements and applicable pollution prevention regulations including Particularly Sensitive Sea Areas (PSSA) requirement if applicable. Arrival port requirement pertaining to compliant fuel use, treated Sewage and Greywater discharge etc obtained by Master. All HOD been informed (E/R, Galley and Deck). |  |  |  |  |  |  |  |
|  | Emission control area (ECA) boundary plotted on chart and duty engineer informed at least three hours prior to entering the ECA for changing over to 0.1% Sulphur limit fuel. |  |  |  |  |  |  |  |
|  | Regulated California Waters (RCW) boundary 24 miles from the coast including islands plotted on chart and duty engineer informed at least three hours prior entering RCW for change over to distillate fuel (MDO and/or MGO) 0.1% Sulphur limit. |  |  |  |  |  |  |  |
|  | ECDIS terminals are setup correctly for navigation in ocean waters with route displayed |  |  |  |  |  |  |  |
|  | Bridge manning appropriate to maintain a proper look-out |  |  |  |  |  |  |  |
|  | GPS positions accuracy verified by bearing and distance lines of position (LOP) using terrestrial objects, by DR/EP plotting as per passage plan. (HDOP value should not exceed 4 for GPS position accuracy). Position fixing intervals / methods – As per passage plan. |  |  |  |  |  |  |  |
|  | Changes in local weather monitored and regular barometer observations made |  |  |  |  |  |  |  |
|  | NAVAREA navigational warning broadcasts and other long range weather reports monitored. ECDIS route check carried out after plotting. |  |  |  |  |  |  |  |
|  | Area reporting systems (e.g. AMVER) participated as appropriate |  |  |  |  |  |  |  |
|  | Advice / Recommendations in sailing directions monitored |  |  |  |  |  |  |  |
|  | Passage Plan been updated as required |  |  |  |  |  |  |  |
|  | Errors of Gyro/Magnetic Compasses being checked at least once per Watch |  |  |  |  |  |  |  |
|  | Keeping a Look-Out being given due priority |  |  |  |  |  |  |  |
|  | Radar performance monitor checked every watch and recorded in deck logbook. |  |  |  |  |  |  |  |
|  | Echo sounder depth compared with the charted depth (in depths less than 100m) and comparison recorded in the deck logbook. |  |  |  |  |  |  |  |
|  | AIS accuracy of the target information checked by comparing radar range and bearing of the same target. Result recorded in deck logbook. |  |  |  |  |  |  |  |
|  | Preparations been made for landfall |  |  |  |  |  |  |  |
|  | Autopilot off course alarm tested ( Refer to maker manual for testing procedures) |  |  |  |  |  |  |  |
|  | Speed log accuracy checked by comparing with GPS |  |  |  |  |  |  |  |
|  | Other Checks |  |  |  |  |  | YES | NA |