| VESSEL |  |  | DATE: | Click or tap to enter a date. | VOY NO: |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Laminated copy of the checklist shall be maintained in wheel house and compliance will be recorded in deck log book for each anchoring and anchor watch. |  |  |  |  |  |  |  |  |
| SN | DESCRIPTION |  |  |  |  |  | YES | NA |
|  | Anchoring Appraisal and Planning |  |  |  |  |  |  |  |
| 1 | Anchoring plan checked and approved by the Master |  |  |  |  |  | ☐ | ☐ |
| 2 | Anchoring position identified taking into account: |  |  |  |  |  | ☐ | ☐ |
| 2.1 | - Availability of appropriate space / searoom at the anchorage |  |  |  |  |  | ☐ | ☐ |
| 2.2 | - Proximity of navigational hazards like pipelines, submarine cables etc |  |  |  |  |  | ☐ | ☐ |
| 2.3 | - Advice in sailing directions and other publications and practice of the port |  |  |  |  |  | ☐ | ☐ |
| 2.4 | - Traffic density in vicinity and availability of swinging room |  |  |  |  |  | ☐ | ☐ |
| 2.5 | - Draft, Depth of water and company UKC requirements |  |  |  |  |  | ☐ | ☐ |
| 2.6 | - Availability of suitable Landmarks |  |  |  |  |  | ☐ | ☐ |
| 2.7 | - Anchor to be used and scope of anchor cable required / available |  |  |  |  |  | ☐ | ☐ |
| 2.8 | - Suitable seabed type and holding conditions |  |  |  |  |  | ☐ | ☐ |
| 2.9 | - Direction strength of wind and current |  |  |  |  |  | ☐ | ☐ |
| 2.10 | - Weather conditions and available shelter |  |  |  |  |  | ☐ | ☐ |
| 3 | Tidal height checked to confirm sufficient water available for the duration of the anchorage |  |  |  |  |  | ☐ | ☐ |
| 4 | Tidal stream checked with particular reference to effect on slow speed manoeuvring |  |  |  |  |  | ☐ | ☐ |
| 5 | Anchor lashings removed, cables and windlass checked and ready for use |  |  |  |  |  | ☐ | ☐ |
| 6 | Marine superintendent informed if anchoring in deep waters ( more than 80 M depth) |  |  |  |  |  | ☐ | ☐ |
| 7 | Engine room and anchor party informed of the time of anchoring. Tool box meeting carried out with anchor party and use of PPE as per company matrix discussed |  |  |  |  |  | ☐ | ☐ |
| 8 | Intended anchor position of the ship reported to the port authority |  |  |  |  |  | ☐ | ☐ |
| 9 | Lights, shapes and sound signalling apparatus checked and ready for use |  |  |  |  |  | ☐ | ☐ |
| 10 | Reduction to manoeuvring speed in ample time |  |  |  |  |  | ☐ | ☐ |
| 11 | Security measures required by the Ship Security Plan (SSP) |  |  |  |  |  | ☐ | ☐ |
| 12 | Walkie talkie charged and communication established with anchor party |  |  |  |  |  | ☐ | ☐ |
| 13 | Main engine tried out for astern movement and kept ready for manoeuvring |  |  |  |  |  | ☐ | ☐ |
|  | Anchor watch While at Anchor the OOW Should: |  |  |  |  |  |  |  |
| 1 | Anchoring position and anchor bearing been logged and plotted on chart |  |  |  |  |  | ☐ | ☐ |
| 2 | Turning circle marked on chart and found safe |  |  |  |  |  | ☐ | ☐ |
| 3 | Anchor watch established and effective radar watch / proper look-out maintained |  |  |  |  |  | ☐ | ☐ |
| 4 | Appropriate lights and shape exhibited and appropriate sound signals made in accordance with all applicable regulations (as required) |  |  |  |  |  | ☐ | ☐ |
| 5 | Appropriate flags displayed |  |  |  |  |  | ☐ | ☐ |
| 6 | Ships in vicinity plotted/monitored |  |  |  |  |  | ☐ | ☐ |
| 7 | Masters standing orders / Night orders complied with |  |  |  |  |  | ☐ | ☐ |
| 8 | Anchor bearings checked regularly using fixed navigation marks or readily identifiable shore objects at sufficiently frequent intervals to ensure vessel is securely anchored |  |  |  |  |  | ☐ | ☐ |
| 9 | Company UKC requirement complied with at anchorage |  |  |  |  |  | ☐ | ☐ |
| 10 | Anchor watch alarm on GPS / ECDIS established and monitored |  |  |  |  |  | ☐ | ☐ |
| 11 | Radar guard rings , echo sounder, ECDIS alarms monitored |  |  |  |  |  | ☐ | ☐ |
| 12 | AIS status updated and AIS monitored |  |  |  |  |  | ☐ | ☐ |
| 13 | Weather and tide information available - meteorological, tidal condition and state of the sea being observed |  |  |  |  |  | ☐ | ☐ |
| 14 | Vessel’s swinging circle boundary monitored. In tidal waters, tide timings written & vessel’s swing monitored |  |  |  |  |  | ☐ | ☐ |
| 15 | Other ships alerted if anchoring in close proximity with own vessel |  |  |  |  |  | ☐ | ☐ |
| 16 | VHF watch maintained on appropriate channel for port, agent and traffic |  |  |  |  |  | ☐ | ☐ |
| 17 | Hawse pipe cover properly secured |  |  |  |  |  | ☐ | ☐ |
| 18 | Vessel's deck and over side well illuminated at night |  |  |  |  |  | ☐ | ☐ |
| 19 | Vessel access control precautions are maintained in respect of vessel security. No unauthorized persons permitted to board vessel |  |  |  |  |  | ☐ | ☐ |
| 20 | Measures been taken to monitor boats, crafts approaching or operating in the vicinity ,considering threat of piracy and armed robbery |  |  |  |  |  | ☐ | ☐ |
| 21 | Fire/safety and security rounds taken regularly by duty AB, with radio reporting schedule to bridge as instructed by OOW |  |  |  |  |  | ☐ | ☐ |
| 22 | Main engines and machinery available in accordance with Master's instructions |  |  |  |  |  | ☐ | ☐ |
| 23 | Master to be notified as per master’s orders and necessary measures taken if visibility or weather deteriorates or wind speed or swell picks up |  |  |  |  |  | ☐ | ☐ |
| 24 | Master to be notified and immediate action if other vessel approaches in close proximity and there is danger of collision |  |  |  |  |  | ☐ | ☐ |
| 25 | Master and engine room to be notified and necessary measures undertaken if the ship drags anchor |  |  |  |  |  | ☐ | ☐ |
| 26 | Measures been taken to protect the environment from pollution by the ship and to comply with applicable pollution regulations |  |  |  |  |  | ☐ | ☐ |
| 27 | Navtex / EGC messages monitored |  |  |  |  |  | ☐ | ☐ |
| 28 | Any oil floating around vessel reported |  |  |  |  |  | ☐ | ☐ |
| 29 | Regular inspections of the anchoring equipment made . Deck watch to confirm, at regular intervals, that the windlass brake is holding, chain stopper in place, cable has not moved and is not vibrating |  |  |  |  |  | ☐ | ☐ |
| 30 | BNWAS kept operational in manual mode |  |  |  |  |  | ☐ | ☐ |
| 31 | Anchor marked using a flag for easy identification by the Bridge officer against slipping of chain |  |  |  |  |  | ☐ | ☐ |
|  | Other checks |  |  |  |  |  |  |  |
| 1. | If Wind force is exceeding BF 5 or there is a forecast for deteriorating weather within the next 12 hours, Master to inform port authority and pick up anchor and drift at safe location |  |  |  |  |  | ☐ | ☐ |