| VESSEL |  |  | DATE: |  | VOY NO: |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Laminated copy of the checklist shall be maintained in wheel house and compliance will be recorded in deck log book for each watch during restricted visibility. |  |  |  |  |  |  |  |  |
| SN | DESCRIPTION |  |  |  |  |  | YES | NA |
|  | Master and Engine room informed (as required in Master’s Standing Orders and the SMS) and engines kept ready for manoeuvring |  |  |  |  |  |  |  |
|  | Both Radars, ARPA / plotting facilities kept ON, controls like gain, tuning properly set and long range scanning carried out. All targets tracked. Radar performance checked |  |  |  |  |  |  |  |
|  | Helmsman on stand-by. Vessel on hand Steering as required and additional steering motor kept ON (where possible) |  |  |  |  |  |  |  |
|  | Fog Signalling Apparatus kept ON and sounded at appropriate intervals |  |  |  |  |  |  |  |
|  | Navigation Lights kept ON |  |  |  |  |  |  |  |
|  | VHF kept ON and watch maintained on CH 16 and the local working channel, if applicable |  |  |  |  |  |  |  |
|  | All watertight doors and openings kept closed |  |  |  |  |  |  |  |
|  | Echo sounder kept ON , in depth less than 100 meters |  |  |  |  |  |  |  |
|  | All available navigational aids including AIS used |  |  |  |  |  |  |  |
|  | Additional lookouts been posted in suitable position |  |  |  |  |  |  |  |
|  | Manning level increased on bridge for duration of restricted visibility |  |  |  |  |  |  |  |
|  | COLREGS being complied with, particularly with regard to rule 5,6,19 and 35 Rule 19 – conduct of vessels in restricted visibility Rule 35 – sound signals in restricted visibility Rule 5 – look-out Rule 6 – safe speed |  |  |  |  |  |  |  |
|  | Ship on safe speed and ready to reduce speed, stop , reverse propulsion or turn away from danger |  |  |  |  |  |  |  |
|  | Possibility of anchoring been considered if ships position is in doubt and vessel in suitable depth |  |  |  |  |  |  |  |
|  | Chipping / Any work involving noise near bridge stopped |  |  |  |  |  |  |  |
|  | Frequency of position fixing increased if in proximity of shallow waters / dangers |  |  |  |  |  |  |  |
|  | Anchor lashings removed if vessel is within anchoring depths (as per masters discretion) |  |  |  |  |  |  |  |
|  | Masters additional standing instructions / night orders complied with |  |  |  |  |  |  |  |
|  | Parallel index techniques, radar ranges and bearings used when monitoring the passage in coastal and pilotage waters |  |  |  |  |  |  |  |
|  | Other ship specific checks : |  |  |  |  |  | YES | NA |