| VESSEL |  |  | DATE: | Click or tap to enter a date. | VOY NO: |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Laminated copy of the checklist shall be maintained in wheelhouse and compliance will be recorded in deck logbook |  |  |  |  |  |  |  |
| SN | DESCRIPTION |  |  |  |  | YES | NA |
|  | Master, engine room and crew been informed about expected bad weather and estimated time of start of bad weather |  |  |  |  | ☐ | ☐ |
|  | Chief Engineer informed that reduction in speed may be required, depending on severity of weather encountered. Engines to be ready for manoeuvring |  |  |  |  | ☐ | ☐ |
|  | Speed and Course been adjusted as necessary or avoiding action taken, as the case may be, to ease the stress of weather on the vessel |  |  |  |  | ☐ | ☐ |
|  | Ship manoeuvred to minimise the risk of broaching, pooping and/or synchronous rolling |  |  |  |  | ☐ | ☐ |
|  | Navigational watch strengthened |  |  |  |  | ☐ | ☐ |
|  | Appropriate meteorological instruments, weather reports, monitored at regular intervals and compared with the weather actually being experienced |  |  |  |  | ☐ | ☐ |
|  | Two hourly log-entries of meteorological conditions prevailing, and vessel’s behaviour made |  |  |  |  | ☐ | ☐ |
|  | Weather Reports transmitted to the appropriate authorities, or, in the case of Tropical Storms, danger messages transmitted in accordance with SOLAS |  |  |  |  | ☐ | ☐ |
|  | Crew been warned to avoid Upper Deck areas made dangerous by weather |  |  |  |  | ☐ | ☐ |
|  | All personnel have been specifically instructed not to go out on deck without permission of the Master |  |  |  |  | ☐ | ☐ |
|  | Safety Lines/Hand Ropes been Rigged where necessary |  |  |  |  | ☐ | ☐ |
|  | Safe access to bow rigged & ready |  |  |  |  | ☐ | ☐ |
|  | All scupper holes/freeing ports on deck kept clear. |  |  |  |  | ☐ | ☐ |
|  | Slack ballast tanks pressed-up / pumped out, to eliminate sloshing damage in tanks. Heavy weather ballast taken on commencement of voyage or in ample time prior onset of heavy weather |  |  |  |  | ☐ | ☐ |
|  | Draft, trim, GM, stress, sloshing, propeller immersion adjusted for better handling of the vessel in heavy weather. |  |  |  |  | ☐ | ☐ |
|  | All loose and movable objects on deck, engine room, galley, cabins and accommodation and in storerooms properly secured |  |  |  |  | ☐ | ☐ |
|  | Ship’s accommodation been secured, and all Ports and Deadlights closed |  |  |  |  | ☐ | ☐ |
|  | All watertight doors shut/Watertight integrity checked |  |  |  |  | ☐ | ☐ |
|  | All closable vents on deck, cargo hatches / holds, forecastle, poop and around accommodation house have been checked closed and tight. Special attention paid to bunker tank vents and sounding pipes. Save all plugs removed |  |  |  |  | ☐ | ☐ |
|  | Lashings and dogs of doors and hatches tightened. Hatch cover locking devices rechecked |  |  |  |  | ☐ | ☐ |
|  | Dampers for cargo hold ventilation closed |  |  |  |  | ☐ | ☐ |
|  | An internal means of securing the forecastle hatch rigged, such as a chain block providing a downward pull to hold the cover in place in the event that the dogs are sheared or displaced |  |  |  |  | ☐ | ☐ |
|  | Cable locker access doors and bitter end checked and confirmed tight; cable locker and forecastle space educator confirmed clear and operational |  |  |  |  | ☐ | ☐ |
|  | All portable items in the forecastle space secured against movement and all rags, etc likely to block eductor suctions removed. Bilge well not over stowed |  |  |  |  | ☐ | ☐ |
|  | Control boxes for winch and valve operating levers, where provided, properly sealed and secured |  |  |  |  | ☐ | ☐ |
|  | All steel WT doors / booby hatches / engine room skylight /fore peak store /steering gear room been closed and tightened up. |  |  |  |  | ☐ | ☐ |
|  | All cargo lashings checked / tightened / doubled up as required. Cargo lashing equipment not in use secured |  |  |  |  | ☐ | ☐ |
|  | Anchors have been checked right up into hawse pipes with no free movement and additional wire lashings taken and tightened. Cable stoppers correctly set. |  |  |  |  | ☐ | ☐ |
|  | Spurling pipe steel-covers checked in position, cemented over / covered |  |  |  |  | ☐ | ☐ |
|  | All mooring ropes on the mooring winches secured and covered |  |  |  |  | ☐ | ☐ |
|  | All gangways/accommodation ladders properly secured, and gangway motors covered with canvas |  |  |  |  | ☐ | ☐ |
|  | Pilot ladders and hoists lashed and secured, and additional lashings taken as required |  |  |  |  | ☐ | ☐ |
|  | Lifeboats /life rafts / Embarkation ladders properly secured. Lifeboat equipment checked for lashings |  |  |  |  | ☐ | ☐ |
|  | Portable deck equipment, including forward life raft and boarding ladder, lifebuoys, fire hoses and nozzles, pilot ladder(s) etc moved to safe storage location |  |  |  |  | ☐ | ☐ |
|  | Deck cranes / grabs / Provision stores cranes / engine room cranes /bunker davit etc properly secured. Lashings rechecked |  |  |  |  | ☐ | ☐ |
|  | Pipelines /Stores / spares, equipment, etc. stowed on deck secured |  |  |  |  | ☐ | ☐ |
|  | Steering gear room checked, and additional lashings applied as necessary |  |  |  |  | ☐ | ☐ |
|  | Special attention must be paid to securing paint, lube oil drums and chemical stores. All oil drums shall be properly and tightly lashed using wires and not ropes |  |  |  |  | ☐ | ☐ |
|  | All crockery, cutlery, tables, chairs, secured in the mess rooms / pantries / stores |  |  |  |  | ☐ | ☐ |
|  | Chief cook notified and provision rooms (cold and dry) galley and mess rooms (crew and officers) secured |  |  |  |  | ☐ | ☐ |
|  | All recreational equipment such as TV / VCR / Music Systems / Gymnasium equipment properly secured. Additional lashings on PC’s, monitors, copy machines and printers taken |  |  |  |  | ☐ | ☐ |
|  | Bridge prepared for rough weather |  |  |  |  | ☐ | ☐ |
|  | Vessel on hand steering and both steering motors kept ON |  |  |  |  | ☐ | ☐ |
|  | Oxygen acetylene bottles rechecked and lashed and in appropriate place |  |  |  |  | ☐ | ☐ |
|  | Crew provided sufficient rest to ensure compliance with MLC rest hour requirements |  |  |  |  | ☐ | ☐ |
|  | Watch keepers medicated if prone to sea sickness |  |  |  |  | ☐ | ☐ |
|  | OTHER SHIP SPECIFIC CHECKS |  |  |  |  |  |  |
|  | Crew shall remain inside accommodation |  |  |  |  | ☐ | ☐ |
|  | 440 volt power supply for deck machinery and forecastle store machinery put off |  |  |  |  | ☐ | ☐ |
|  | Luboil level in machineries to be checked and topped up as required |  |  |  |  | ☐ | ☐ |