| SN | Occasions to Call the Master (As required by the SMS, Master’s Standing Orders and night orders including the following). Laminated copy to be maintained in wheelhouse. |
| --- | --- |
|  | If restricted visibility is encountered or expected |
|  | If traffic conditions, density or the movements of other ships including concentration of fishing vessels or if the configuration of radar plots are causing concern |
|  | When a distress alert has been received or a distress signal has been sighted |
|  | If difficulties are experienced in maintaining course due to heavy traffic, weather, sea etc |
|  | When there is a significant difference between the latest observed position and the expected position of the ship |
|  | On failure to sight land, a navigation mark or obtain soundings by the expected time |
|  | If, unexpectedly, land or a navigation mark is sighted or an unexpected change in soundings occurs |
|  | If amendments to the passage plan require immediate approval |
|  | If there is a breakdown of the engines, propulsion machinery remote control, steering gear or any essential navigational equipment, alarm or indicator |
|  | If the communications or GMDSS radio equipment malfunctions |
|  | In heavy weather, if any doubt about the possibility of weather damage |
|  | If the ship meets any hazard to navigation, such as ice or a derelict or unlit craft |
|  | If any vessel security concerns arise |
|  | In any emergency or close quarter situation |
|  | If in doubt of vessels position |
|  | If there is a reduction in speed or if there is any circumstance that may affect the ETA |
|  | When the soundings disagree with the soundings on the chart along vessel’s track, or any other indication of shallow water not on the chart |
|  | If the OOW observes any sudden change in the sea, such as sudden swell, or discoloration which may indicate shoals or other danger |
|  | When a significant change of weather parameters is noticed such as sudden shift in wind direction and/or increase in wind speed or if barometer pressure drops significantly |
|  | If any of the signs associated with a tropical revolving storm are observed |
|  | If any navigation warning or weather information is received which can affect safety of vessel |
|  | If the OOW is feeling unwell or fatigued or unable to stay alert for any reason whatsoever or any serious irregularity in preparing to relieve or surrender the watch |
|  | If oil is seen around the vessel from whatever source |
|  | If OOW is having difficulty in coping with monitoring ship’s position and collision avoidance functions simultaneously |
|  | When the OOW feels the need to have additional bridge team resources |
|  | When point or time the Master instructed is reached |
|  | When an incident has occurred resulting in any injury to personnel or damage to equipment |
|  | If any damage, suspected damage or abnormal condition of cargo is noticed |
|  | In any cases when the situation is beyond the experience or control of the OOW or if there is any doubt regarding the safety of the ship, or prevention of pollution or ability to comply with regulatory requirements |
|  | When a "give way vessel" comes within 15 minutes of the allowable CPA without significant bearing change, and all communication attempts to determine that vessel’s intentions have failed |

If the Master needs to be called, particularly where there is concern about the safety of the ship, this should be done early enough to allow the Master sufficient time to understand and respond effectively to the situation.

Failing to call the Master in a timely manner can lead to an increased level of risk in relation to:

- Collision

- Grounding

- Safety of life

- Damage to the environment

- Vessel delays

- Cargo leaks or spills

- Property damage

- Commercial losses; or

- Reputation losses due to delays or damage.