| VESSEL |  |  | DATE: | Click or tap to enter a date. | VOY NO: |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Laminated copy of the checklist shall be maintained in wheel house and compliance will be recorded in deck log book for each watch. |  |  |  |  |  |  |  |
| SN | DESCRIPTION |  |  |  |  | YES | NA |
|  | COLREGS Rule 10 Traffic Separation Schemes complied with |  |  |  |  | ☐ | ☐ |
|  | All of the other COLREG rules complied with when vessel is in TSS |  |  |  |  | ☐ | ☐ |
|  | Local regulations complied with, in addition to the COLREGS |  |  |  |  | ☐ | ☐ |
|  | If not using a traffic separation scheme, avoid it by as wide a margin as is practicable |  |  |  |  | ☐ | ☐ |
|  | Passage plan reviewed and route planned in the appropriate traffic lane in the general direction of traffic flow for that lane |  |  |  |  | ☐ | ☐ |
|  | Route kept clear of a traffic separation line or separation zone |  |  |  |  | ☐ | ☐ |
|  | Route planned to join or leave a traffic lane at the termination of the lane (if joining or leaving from either side, it shall be as small an angle to the general direction of traffic flow as practicable) |  |  |  |  | ☐ | ☐ |
|  | Route planned to avoid crossing the traffic lanes |  |  |  |  | ☐ | ☐ |
|  | Route planned not to enter a separation zone or cross a separation line |  |  |  |  | ☐ | ☐ |
|  | Precautions taken when navigating in areas near the terminations of traffic separation schemes |  |  |  |  | ☐ | ☐ |
|  | Route planned to avoid inshore traffic zones |  |  |  |  | ☐ | ☐ |
|  | Route planned not to anchor in a traffic separation scheme or in areas near its terminations |  |  |  |  | ☐ | ☐ |
|  | All TSS associated traffic reporting requirements, whether compulsory or voluntary complied with and recorded |  |  |  |  | ☐ | ☐ |
|  | Monitor the bridge radio and any instructions received from a VTS station complied only if it is in accordance with the COLREGS |  |  |  |  | ☐ | ☐ |
|  | Over taking avoided – especially towards the end of the scheme |  |  |  |  | ☐ | ☐ |
|  | Look out maintained for fishing vessels |  |  |  |  | ☐ | ☐ |
|  | Look out maintained for crossing vessels and other vessels that may attempt some unconventional activities entering and exiting these schemes |  |  |  |  | ☐ | ☐ |
|  | Bridge manning level increased as required at focal points in TSS (e.g. crossing area/caution area). |  |  |  |  | ☐ | ☐ |
|  |  |  |  |  |  | ☐ | ☐ |
|  |  |  |  |  |  | ☐ | ☐ |