| VESSEL |  |  | DATE: | Click or tap to enter a date. | VOY NO: |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Laminated copy to be maintained and compliance to be recorded in deck logbook prior each use of pilot ladder. |  |  |  |  |  |  |  |  |
| SN | DESCRIPTION |  |  |  |  | YES | NO | NA |
|  | Pilot ladder certified as per SOLAS, CH V, Reg 23 and IMO RES A 1045(27). Maker certificate on board and maker name plate clearly visible with matching certification |  |  |  |  | ☐ | ☐ | ☐ |
|  | Each pilot ladder clearly marked “For Marine Transfers only” |  |  |  |  | ☐ | ☐ | ☐ |
|  | Pilot ladder shall be clean of grease, oil, chemical, paint etc and safe to use |  |  |  |  | ☐ | ☐ | ☐ |
|  | Steps, spreaders, chocks, side ropes, manropes, securing ropes, heaving lines, thimbles, eye pads etc in good condition and inspected prior use |  |  |  |  | ☐ | ☐ | ☐ |
|  | Pilot transfer area painted with non-slip paint, clean and free of obstructions. Trip hazards in the pilot transfer area highlighted with contrasting paint |  |  |  |  | ☐ | ☐ | ☐ |
|  | Pilot ladder shall be less than 30 months from date of manufacture |  |  |  |  | ☐ | ☐ | ☐ |
|  | Manrope and associated ropes for securing the ladder on deck shall be less than 12 months from date of manufacture (certificates held onboard) |  |  |  |  | ☐ | ☐ | ☐ |
|  | Lifebuoy with self-igniting light shall be available at the pilot boarding area |  |  |  |  | ☐ | ☐ | ☐ |
|  | Pilot ladder marked at one meter throughout the length of the ladder to facilitate the rigging of the ladder to the required height and is clearly visible |  |  |  |  | ☐ | ☐ | ☐ |
|  | Crew wearing proper PPE including life vest, lifeline, harness etc, and working overside requirements shall be complied with |  |  |  |  | ☐ | ☐ | ☐ |
|  | Visual draft shall be checked before pilot boarding and freeboard shall be less than 9M when only pilot ladder is used. Loadicator draft shall not be relied upon for freeboard. Pilot ladder is lowered 1.5 metres above water level OR as instructed by pilot. Precautions must be taken to avoid the boat making contact and damaging ladder steps |  |  |  |  | ☐ | ☐ | ☐ |
|  | Pilot ladder properly secured to the deck of ship at strong point using rope (rolling hitch knot). Shackles not used on the side rope of the ladder |  |  |  |  | ☐ | ☐ | ☐ |
|  | Manila man ropes (if required) are to have a diameter of at least 28mm and no more than 32mm and securely rigged to strong point on deck and pass through the eye on handhold stanchions |  |  |  |  | ☐ | ☐ | ☐ |
|  | Pilot ladder resting on parallel body of ship with ladder steps horizontal and chocks tightly secured |  |  |  |  | ☐ | ☐ | ☐ |
|  | Tripping line may not be attached to the bottom most step. Tripping line if used, must lead forward to avoid fouling with Pilot launch |  |  |  |  | ☐ | ☐ | ☐ |
|  | Lower platform of the combination ladder in horizontal position and secured to the ship's side. Lower platform minimum of 5 metres above the sea level. Angle of slope not > 45° |  |  |  |  | ☐ | ☐ | ☐ |
|  | Each step of the pilot ladder rests firmly against the ship’s side. The pilot ladder secured to ship’s side 1.5 meters above accommodation platform using eye pad or handhold or by a mechanical device such as magnetic clamps or a pneumatic suction pad |  |  |  |  | ☐ | ☐ | ☐ |
|  | Boarding area adequately lit for pilot transfers at night |  |  |  |  | ☐ | ☐ | ☐ |
|  | Pilot ladder rigged as per International Maritime Pilots Association "Required boarding arrangements for pilots" poster |  |  |  |  | ☐ | ☐ | ☐ |
|  | The vessel shall provide a good lee on the side where ladder is rigged. Course steered to avoid rolling / pitching |  |  |  |  | ☐ | ☐ | ☐ |
|  | Proper communication established between the Bridge, officer on deck and the pilot boat |  |  |  |  | ☐ | ☐ | ☐ |
|  | The rigging of the pilot transfer arrangement supervised by a responsible officer maintaining communication with the bridge. The officer shall ensure the ladder is properly secured. Pilot ladder and combination ladder (if required) tested prior to use |  |  |  |  | ☐ | ☐ | ☐ |
|  | At least two people (including one Officer) available near the pilot boarding area to assist pilot's embarkation / disembarkation. Crew appointed to escort the pilot by a safe route to and from the navigation bridge |  |  |  |  | ☐ | ☐ | ☐ |