| VESSEL |  |  | DATE: |  | VOY NO: |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Laminated copy of the checklist shall be maintained in wheelhouse and compliance will be recorded in deck logbook. Checks are to be carried out daily by the 2NO (preferably noon time). |  |  |  |  |  |  |  |
| SN | DESCRIPTION |  |  |  |  | YES | NA |
|  | All clocks, Bridge-E/room, Master Clock System synchronized with GPS |  |  |  |  |  |  |
|  | Forward and aft whistle tested on manual and electric mode (Not in poor visibility / when Traffic is nearby) |  |  |  |  |  |  |
|  | Magnetic compass checked for bubbles from the steering position |  |  |  |  |  |  |
|  | All Navigational lights / NUC lights / signal lights including back up lights and navigation light failure alarm tested |  |  |  |  |  |  |
|  | Navigation light space doors securing checked |  |  |  |  |  |  |
|  | General emergency alarm tested |  |  |  |  |  |  |
|  | Bridge / ECR main telephone, sound powered telephone and PA system tested |  |  |  |  |  |  |
|  | Telegraph / logger tested |  |  |  |  |  |  |
|  | Aldis lamp tested on main power and battery. Three spare bulbs kept. |  |  |  |  |  |  |
|  | Bridge equipment data loggers checked for sufficient paper |  |  |  |  |  |  |
|  | Window sprinklers / wipers / clear view screen checked for operation |  |  |  |  |  |  |
|  | Steering motor and Telemotor – Changeover / NFU mode steering - Tested |  |  |  |  |  |  |
|  | Dry and wet bulb thermometers including water level, muslin cloth, housing box checked |  |  |  |  |  |  |
|  | AIS checked for correct ships information (Dynamic / voyage / navigation status) |  |  |  |  |  |  |
|  | Passage plan - reviewed and updated as required |  |  |  |  |  |  |
|  | Autopilot checked for effective tuning depending on prevailing weather conditions |  |  |  |  |  |  |
|  | Integrity of information displayed on navigation equipment verified (Check for spoofing, jamming HDOP values etc) |  |  |  |  |  |  |
|  | Bridge panels checked for any alarms or fault alarms |  |  |  |  |  |  |
|  | Operational settings and alarms correctly set and checked on bridge equipment as applicable (BNWAS, AIS, GPS anchor watch alarm, XTD alarm etc) |  |  |  |  |  |  |
|  | Configuration settings checked and confirmed to be in accordance with SMS and passage plan (example ARPA true/relative motion, Head up/ North up etc, ECDIS display settings like standard display, all other information, Configuring ECDIS to cross safety contour, parallel indexing etc) |  |  |  |  |  |  |
|  | Bridge equipment functioning properly and successfully communicating with other bridge system to which it is connected (example verify echosounder depth on ECDIS, Navtex messages on ECDIS, GPS position on ECDIS, gyro heading on ECDIS, speed log reading on ECDIS, ARPA) |  |  |  |  |  |  |
|  | Buzzer / lamp tests / Dimmer lights on each equipment (as applicable) tested. |  |  |  |  |  |  |
|  | Rudder / RPM indicators / wind indicator checked |  |  |  |  |  |  |
| 24 | CCTV operational with all cameras functioning. |  |  |  |  |  |  |